Last Revised: April 17, 2026

Salting Out Protocol for clean high molecular weight DNA

Original Primary author(s): Chris Faulk, Beth Thielen, Bart Theelen

Version Author: Kristen Bastug, Fredrickson Wasswa

Purpose: Cheap, non-toxic method to extract DNA from cultured bacterial pellet to provide high-quality DNA without expensive spin columns. Generates high molecular weight for long-read sequencing.

Note: This protocol does not utilize RNAse, but this can be added

Reagent preparation

|  | Reagent | Vol 1 sample | Vol 300 Samples | Vol _____ Samples |
| --- | --- | --- | --- | --- |
|  | Salting out lysis buffer: 10mM Tris-HCl, 2mM EDTA, 400mM NaCl, 2% SDS | 440 uL | 1320 mL |  |
|  | 5M NaCl (AG Scientific, S-2440-500ml, room temp) | 300 uL | 90 mL |  |
|  | Fresh 70% EtOH (room temp) | 1.6 mL | 480 mL |  |
|  | 100% Isopropanol (room temp) | 800 uL | 240 mL |  |
|  | Low TE elution buffer (or molecular grade water) | 61 uL | 18.3 mL |  |
|  | DNA/RNA Shield (Zymo, cat R1100-50, store room temp) | 50 uL | 150 mL |  |
|  | Proteinase K (invitrogen, store -20 C) | 10 uL | 3 mL |  |
|  | PBS, 1x (Corning, 21-040-CV, room temp) | 1 mL | 300 mL |  |

Input specimen: Liquid specimens

Alternatives: < 10 mg tissue stored in DNA shield for at least 1 hour, dry tissue or use in <50 ul of DNA shield total volume.

Extraction procedure:

- Ensure the lysis buffer is fully dissolved, no crystals or precipitates should be visible. This can be accomplished by 10-15 mins incubation in a 37 ⁰C water bath.

- Centrifuge the liquid samples for 20 mins at 3200 G (or Max speed)

- Pipette off supernatant and discard

- Resuspend the pellet in 1mL PBS

- Repeat centrifugation for 20 mins at 3200 G

- Pipette off supernatant and discard

- Dissolve the bacterial cell pellet in 50 ul DNA/RNA shield and make sure it is homogenous*.

*DNA/RNA Shield is included in the lysis buffer because the shield is inherently lytic so it will contain free DNA. Homogenizing by vortexing is not always sufficient. Mechanical homogenizing with the bottom end of a sterile inoculation loop may help a lot.

- Incubate for 1 hour at room temp

Stop Point - Freeze Specimen at -20 or -80

Remove specimen from freezer and allow to thaw. Proceed with the following steps:

- For each specimen, add 10 µL proteinase K (20 mg/mL) to 440 µL of salting out lysis buffer for each specimen

If extracting multiple specimens, can prepare a master mix at this step

| Reagent | 1 Sample | _______Samples |
| --- | --- | --- |
| Proteinase K | 10 uL |  |
| Lysis Buffer | 440 uL |  |

- Add the combined 450 µL of lysis buffer+proteinase K to the bacterial pellet

- Incubate at 55 ⁰C on a shaking block at 500 RPM for 2 hours

  - Start time_________

- Cool to room temperature

- Add 300 µL 5M NaCl and mix thoroughly by inverting

- Spin at max speed in a tabletop microfuge for 10 minutes

  - Start time _________

Precipitation may not always result in fully precipitated pellet, with flakes still in solution. If this happens, transfer the supernatant into a fresh tube leaving behind as much of the precipitate as possible and repeat the centrifugation step

- Transfer the supernatant in new tube (this has your DNA)

- Add a 1:1 ratio (800 uL) of 100% isopropanol to the supernatant (1:1 to your original specimen size). This will precipitate the DNA into solid

- Invert the sample and rock in hands or on rocker for 10 minutes minimum

  - Start time __________

longer improves precipitation

- Spin at max speed in a tabletop microfuge for 10 minutes at room temperature

- Discard supernatant

- Add 800 uL 70% ethanol

- Incubate at 55 ⁰C for 20  minutes .

  - Start time __________

This step is essential to remove salts and give good 260/280 and 260/230 ratios

- Centrifuge at max speed in a tabletop microfuge for 1 minute

- Remove ethanol by pipetting

- Pulse spin in microfuge and let dry to air.

  - Alternatively, you can use a P2 tip to wick away small remaining volumes of ethanol.

- Add 60 µL elution buffer and mix via gentle pipette

  - Water preferred over low TE buffer,a s it avoids adding EDTA, which can be problematic for gel electrophoresis. However, the elution buffer and volume can be changed to suit the specific experimental goal

- Incubate at 55 ⁰C for 10 minutes

  - Start time __________

- Resuspend overnight at 4 ⁰C for full resuspension

Optional Post-Extraction Steps

- Quantify nucleic acid concentration

Refs:

https://www.protocols.io/view/dna-extraction-salting-out-q26g7r151vwz/v4